Task Procedure for ALICE

[ Task Procedure = simplified version of the Specific Plan for Health and Safety, adapted for ALICE Collaboration. - PPSPS = Plan Particulier de Sécurité et de Protection de la Santé ]

Task to be realized	 	Commissioning of Laser system of TPC

Location of work		SXL2, clean room

Dates of work		start 15.4.06			end  31.08.06

Sub-project	TPC				            Institute  GSI Darmstadt+Univ.HD+CERN

Sub-project Leader	Peter Braun-Munzinger	phone		71469

Safety link-person 	Peter Glassel			phone		16 3426

Work-Package Engineer Peter Glassel		phone		16 3426

Local Work supervisor Borge Svane Nielsen 	phone		16 4221

Experimental-Area Manager	Sébastien Evrard/Pieter Ijzermans	phone		16 4821 / 16 4168

Persons involved	Borge Svane Nielsen					physicist

Jorn Westergaard   					technician

Erik K. Olsen      					technician

Gael Renault       					physicist

Jens J. Gaardhoje  					physicist

Niels Lindegaard   					technician			Joachim Baechler					physicist

Jean-Claude Labbe					technician

Brief description of work procedure

- Setting up and operating a class 4 UV-laser (266 nm) with

- The beam line connecting it to the TPC on the clean room floor.

- Alignment of the laser component on both end plates

- Operation of the laser together with FEE readout

| Detailed description (step by step) of the work procedure | Risk assessment | Preventive measures (collective), |
| --- | --- | --- |
| Setting up of laser | UV exposure of eyes, skin | Only people with safety goggles and laser safety training allowed in clean room while setting up. |
| Routine operation of laser | UV exposure of eyes, skin | no risk: beam fully shielded with beam tubes. Laser itself in protection cover. No goggles needed in this phase. Warning signs at clean room entrance. Flashing sign during operation. |
| Alignment of laser optics on end plates | work on scaffold | Regular scaffolds with barriers. Changes of scaffolding beyond moving them on the floor will be certified by B. Moris Not more than 2 people on one scaffold level. |
| Water cooling of laser | water leak | No risk: small flow, sink existing. Cooling only on while in operation. No operation without people present. |
| Operation of TPC with Ne/CO2 and CO2 | leak and suffocation | O2-detectors AND CO2-detctors to be carried by persons inside clean room |

Side and top view of laser

Responsibility

The Work-Package Engineer is responsible of this procedure.

The Work-Package Engineer and the Work-Package work supervisor certify that the persons involved have the proper qualifications and have been properly informed about safety aspects of the work.

WPE		Peter Glassel  …………..	 date	………………….

WPWS	Joachim Baechler ………….	 date	………………….

Approval

E.A.M. (also TSO)	Sébastien Evrard	date	…………...	...................................

16 4821

Safety Coordinator  	Emmanuel Paulat	date	…………...	………………………

16 3870

TS Transport Coord.	Jacky Bernard		date	…………...	………………………

16 0583

If needed, also sent to:

Integration & Installation Coordinator, 	Lars Leistam		16 0551

GLIMOS 					Marc Tavlet		16 0217 ou 70608

Commissioning Coordinator

(and Deputy Glimos),			Hans Taureg		16 0359

SC Inspector					Hervé Buret		16 0013

This task procedure must be on display at the place of work.