Task Procedure for ALICE

[ Task Procedure = simplified version of the Specific Plan for Health and Safety, adapted for ALICE Collaboration. - PPSPS = Plan Particulier de Sécurité et de Protection de la Santé ]

Task to be realized	 	Commissioning of TPC with FEE and HV

Location of work		SXL2, clean room

Dates of work		start 1.4.06			end  31.08.06

Sub-project	TPC				            Institute  GSI Darmstadt+Univ.HD+CERN

Sub-project Leader	Peter Braun-Munzinger	phone		71469

Safety link-person 	Peter Glassel			phone		16 3426

Work-Package Engineer Peter Glassel		phone		16 3426

Local Work supervisor Joachim Baechler	 	phone		16 4106

Experimental-Area Manager	Sébastien Evrard/Pieter Ijzermans	phone		16 4821 / 16 4168

Persons involved	Luciano Musa			task, qualification	physicist

Roberto Campagnolo					physicist

Lennart Osterman					technician

Joachim Baechler					physicist

Hansake Gustafsson					physicist

Claude Engster 					engineer

Antoine Junique					technician

Bjorn Halvor Straume					physicist

Joachim Baechler					physicist

Chilo Garabatos					physicist

Peter Glassel						physicist

Uli Frankenfeld					physicist

Rok Pestotnik						physicist

Sorina Popescu					physicist

Marco Boccioli					physicist

Hans Rudolf Schmidt					physicist

Klaus Schossmaier					physicist

Pierre Vande Vyvre					physicist

Jens Wiechula						physicist

Jean-Claude Labbe					technician

Brief description of work procedure

Commissioning of all 36 readout sectors of  the TPC:

1 Connecting and operating LV, HV, DDL, DCS 2.

2. Water cooling for each sector (one or 2 sectors at a time).

3. Water cooling for resistor rods

3. Operating the drift HV (100 kV)

4. Operating the ROC HV (up to 1.6 kV)

3. Use of laser system to produce tracks

Access to sectors is from scaffolding.

| Detailed description (step by step) of the work procedure | Risk assessment | Preventive measures (collective), |
| --- | --- | --- |
| Cabling and uncabling sectors | work on scaffold | Regular scaffolding with barriers. Changes of scaffolding beyond moving them on the floor will be certified by B. Moris |
| Application of drift HV = 100 kV | electrical shock | no risk: fully shielded inside TPC. DCS control. TPC shell is grounded. HV warning sign at entrance. |
| Application of ROC HV up to about 1600 V | electrical shock | no risk: fully shielded inside TPC HV warning sign at entrance. |
| Operation of FEE and low voltage power supplies (<10 V) | fire | fire alarm system |
| Water cooling (HV rod and FEE cards) | water leak | - Underpressure system, no leakage as long as underpressure is there. - The water is in closed circuit with only a limited amount (<100 l) in the system which could be easily taken by the draining system of the clean room in case of failure |
| Operation of TPC with Ne/CO2 and CO2 | leak and suffocation | O2-detectors AND CO2-detctors to be carried by persons inside clean room |

Responsibility

The Work-Package Engineer is responsible of this procedure.

The Work-Package Engineer and the Work-Package work supervisor certify that the persons involved have the proper qualifications and have been properly informed about safety aspects of the work.

WPE		Peter Glassel  …………..	 date	………………….

WPWS	Joachim Baechler ………….	 date	………………….

Approval

E.A.M. (also TSO)	Sébastien Evrard	date	…………...	...................................

16 4821

Safety Coordinator  	Emmanuel Paulat	date	…………...	………………………

16 3870

TS Transport Coord.	Jacky Bernard		date	…………...	………………………

16 0583

If needed, also sent to:

Integration & Installation Coordinator, 	Lars Leistam		16 0551

GLIMOS 					Marc Tavlet		16 0217 ou 70608

Commissioning Coordinator

(and Deputy Glimos),			Hans Taureg		16 0359

SC Inspector					Hervé Buret		16 0013

This task procedure must be on display at the place of work.